Torņu iela 4-IIIC, 203.kab., Riga, LV 1050, Latvia, phone +371 7223144, fax +371 7223830, e-mail: witbo@apollo.lv

Reg. Nr. 40003353498, Account No. LV45UNLA0050001567270

Latvijas Unibanka, Ridzene affiliate/bank code SWIFT UNLALV 2X

The Client:	Ventspils Free Port Authority

Jāņa iela 19, Ventspils, LV 3601

Project Nr.:	Let89-3

Name of the Project:                             TECHNICAL PROJECT OF NEW BRIDGE

AND ACCESS ROADS AT VENTSPILS

Stage of construction project:	Technical project

interim report 1

Volume:	Volume 1

GT – part

ŪKT – part /in process/

ELT –part/in process/

Manager of construction Project:	O. Zivtiņš

Author of construction project:	SIA ”Witteveen+Bos Latvia”

2007

Composition of project

Technical project of new bridge and access roads at Ventspils

Volume 1 	GT – road part

ŪKT – part of water supply and sewerage external network

ELT – part of electrical supply and communication external netwotrk

Volume 2 	BK – building structures

Volume 3	summary of main works

Contents

Composition of project 2 Contents 3 GT - ROAD PART 4 Explanatory note for GT part 5 General part 5 General instructions 5 Road part 6 Drainage of rain water 7 Facilities and improvement 7 Calculation of perspective intensity of traffic 8 Calculation of coverage of road 9 Characteristics, materials and requirements of strength 11 List of points to be outlined 16 Summary of work amount 19 Contents of drawings 21 GT – Road part 21

General data 	GT-0 	74

Layout M 1:250	GT-1-1 	75

Layout M 1:250	GT-1-2 	76

Layout M 1:250	GT-1-3 	77

Layout M 1:250	GT-1-4 	78

Layout M 1:250	GT-1-5 	79

Horizontal and vertical planning M 1:250	GT-2-1 	80

Horizontal and vertical planning M 1:250	GT-2-2 	81

Horizontal and vertical planning M 1:250	GT-2-3 	81

Horizontal and vertical planning M 1:250	GT-2-4 	83

Horizontal and vertical planning M 1:250	GT-2-5 	84

Profile lengthwise  	GT-2 	85

Sections 	GT-3-1 	86

Sections	GT-3-2 	87

Construction of culverts 	GT-4 	88

GT  - ROAD PART

Explanatory note for GT part

### General part

GT- road part of the object: "Technical Project of new bridge and of access roads at Ventspils” has ben worked out by „Projekts 3” (number of registration certificate of construction merchandiser Nr. 3423-R) on September 2007.

The designing works have been developed taking into account Construction Norms of Latvia, LVS 190-1:2000 “Road trace”, LVS 190-2:1999 “Technical classification, parameters, normal profiles of roads”, LVS 190-3:1999 “One level junction of roads”, as well as other standards and European Norms. (EN).

For designing is used graphical system of road designs AutoCAD Civil 3D 2008.

- Manager of Project GT – road part – Mr Ilmārs Gorda - LBS certificate of construction practice Nr. 20-3827.

Layouts are designed in digital system. Survey is made in coordinate system LKS92 and in the Baltic Height system.

### General instructions

Construction works are to be carried out according the technical Project.

The used coordination system – LKS-92, the used height remarks are Height system of the Baltic as of year 1977 according the APN Geodesic service data of Ventspils city.

Before inception of construction works representatives from all interested organisations have to be invited at site for specification of depth and location of existing engineering network.

During the construction works protection and strengthening of engineering networks must be ensured. Excavation at the distance 2m from the engineering networks has to be carried out without mechanisms.

Insertion is carried out from points of poligonometry procession of Ventspils city. Coordinates of support base can be acquired at Ventspils ZD or SIA ‘’Ģeodēzists’’.

During the construction works there has to be observed following order of works:

- Preparatory works;

- Construction of sewerage, water main, rain water drainage and sewerage;

- Construction of communications and electrical cables, reserve pipes and protective pipes;

- Construction of coverage;

- Construction of facilities and greening.

Trenches must be filled with sand grant with clayey admixture lesser than 5% (VSt 25607_83). Height remarks, distances and radius are given in metres, slopes – in per cent.

### Road part

- The street to be designed is based on the sketch design of „Access roads of the right bank for the new bridge over the Venta river, Ventspils” and of „Access roads of the left bank for the new bridge over the Venta river, Ventspils”worked out by SIA „E. Daniševska birojs”. The routs are connected by the „Reconstruction Project of Rūpniecības street” pk. 2+13.82 worked out by SIA „E. Daniševska birojs” by the „Reconstruction Project of Fabrikas street” pk. 10+65.10 worked out by SIA „Firma L4”.

- According to the terms of reference the Project expects to construct new carriageway with pedestrian sidewalks at both sides of the carriageway as well as technical Access roads for the existing railway bridge. The road of the basic carriageway is constructed with bituminous concrete coverage with width of 7 m, but sidewalks – with coverage of paving stone - with width of 1.5m. The Technical Access roads are expected to be covered with grant in the width of 6m and shoulders with width of 0.75m. At the left side of the carriageway pedestrian sidewalk is separated from the carriageway with the green zone with width of 2m. In the whole section of route, except ridge, the shoulders of the carriageway have to be strengthened with the concrete boards. Solutions see in drawing GT-1 „Layout”.

- Construction of coverage of basic carriageway:

- Hot bituminous concrete upper layer  SMA11, h = 4 cm;

- Hot bituminous concrete sub layer ACb22, h = 7 cm;

- Hot bituminous concrete sub layer ACb32, h = 10 cm;

- Mixture of dolomite shiver, h = 30 cm;

- Frost sustainable layer, h = 50 cm.

- Construction of coverage of technical Access roads:

- Coverage of gravel, h = 18cm;

- Frost sustainable layer, hmin = 30 cm;

- Construction of sidewalk coverage:

- Concrete paving stone, h = 6 cm;

- Shiver offal,  h = 3 cm;

- Base of dolomite shiver, h = 12 cm;

- Frost sustainable layer, h = 30 cm.

- The current approach after construction of engineering networks is expected to be renovated in the current width according the width of the approach gates (see pk. 10+45.00).

- Under the road pk. 3+00.00, pk. 8+40.00, as well as under descent pk. 3+87.00 (on the right) there are expected culverts with diameter of Ø1000 and Ø800. The particular locations see in drawing GT-1 „Layout”. The schematic solutions of construction of culverts see in graphical part, drawing GT-5 „Construction of culverts”.

- In the places where coverage of bituminous concrete changes into the coverage of concrete paving stone must be constructed concrete board BR100.20.8. on base of concrete  C16/20.

- In the places of joints of bituminous concrete must be carried out milling of old coverage ~ in the thickness of 4cm, width of 0.5m, therefore the joints are more qualitative.

- In the places where outside the work borders are carried out reconstruction or construction works of communications must be carried out reconstruction works of the coverage by completely renovating the previous construction and coverage.

- In the places where the current geology is formed by materials which are not usable for construction of the coverage, there must be expected complete extraction and taken away, instead working in the frost sustainable layer from sand of medium coarse. Under the construction of the coverage in the whole length of the road there must be expected deck of geo-textile SS LA 30.

### Drainage of rain water

- For the drainage of rain water the coverage is expected to be constructed with all the necessary cross-gradients. In the section from Rūpniecības street till Fabrikas street, as well as in the bridge zone there is expected to construct the closed water drainage system by draining the water to the rain water collector gully. For the roads of technical service which ensures access to the existing railway bridge, the water is expected to be drained into the ditches, and further – into the river Venta. Places where water flows into the river must be strengthened with boulder pavement.

- Between the parallel road which leads to the garages and existing railway bridge (with connection to the main road pk. 9+53.70) it is expected to make water collector with declivity 0%. See in the graphical part of the Project GT-1 „Layout” and GT-2 „Length profile”.

- Slopes of the ditches are to be made and profiled with declivity 1:1.5 except from pk. 3+92.00 – 4+85.00, where declivity of the left slope for the technical service road is variable ≥ 1:1.5.

- Slopes have to be strengthened with soil and by grassing it with perennial lawn.

- For the access of technical service road to the current railway bridge, as well as with the existing garages pk. 10+10.00, in the Project there is expected joined descent pk. 9+53.70. Because there is expected access to the existing garages and to the reception centre of metal there is dismounted great part of the asphalted yard at the both sides of the road where the Access rod of the new bridge connects to the Fabrikas street (from pk. 9+80.00 – 10+50.00). The existing fenced parking yard at the garages pk. 10+05.00 is expected to be dismantled.

### Facilities and improvement

For the organisation of traffic there is expected to establish new road signs. Road signs are to be mounted on zinced metal support; base is concreted with concrete C16/20. There have to be used reflecting road signs of size I according LVS 77-3:2002.

Improvement works have to be carried out after works of construction of coverage.

Greening works have to be carried out after complete completion of the coverage. For the greening there have to be used soil of h=10cm, grassing it with perennial lawn.

To maintain the current communications by installing double protective pipes on the cables. If it is not posible, there must be expected replacement of communications by coordinating it with the Client and SIA „Lattelekom”, SIA”Citrus Solutions”.

### Calculation of perspective intensity of traffic

The perspective intensity of traffic is forecasted by using visual counting data carried out by SIA „Solvers” on year 2005. In Appendix No.1 it is seen that during the period from 6:00 till 21:00 the existing bridge over the Venta river is used by 21 578 cars of which 1215 or 5.63% are trucks.  Considering the distributive schedule of average traffic intensity per day; the fixed flow forms 93.2% of the whole average intensity per year. Therefore – full intensity of day and night on the bridge is:

GVDI2005 = 23 152 Am/24h (cars/hour)

Therefore total number of vehicles which cross the Venta river during the time period from 21:00 till 6:00 is 1574 Am/9h.

To determine perspective traffic intensity on the new bridge by using distributive schedule of traffic flows of junction No. 9 (Lielais prospect, Bridge, Rūpniecības street), it is calculated than during the period from 6:00 till 21:00 the new bridge can be used by 5091 Am/15h. The result is formed by sum of flow number 4 (Bridge – Rūpniecības street) and flow number 5 (Rūpniecības street – bridge).

Considering the perspective turnover of cargo in the section between new and old bridge over the venta river, it is assumed that lifting of the existing bridge would be necessary once a day. The existing bridge can be closed 20 to 30 minutes. At that time 2nd flow (Lielais prospect – bridge) and 3rd flow (Bridge – Lielais prospect) at 100% use the new bridge over the Venta river. Taking into account traffic intensity during the rush hours it is assumed that during the period from 6:00 till 21:00 the new bridge additionally can be used by 643 Am/15h.

Additional load on new bridge will also be caused by the reconstruction period of old bridge, which is assumed to be one year. Considering the traffic intensity during the bridge reconstruction till the period of year 2027, there have to be calculated of average of 985 Am/15h.

Therefore traffic intensity on the new bridge in year 2005 during the period from 6:00 till 21:00 is composed:

VI2005, for the new bridge  = 5091+ 643 + 985 = 6719 Am/15h

, but

GVDI2005, for the new bridge  = 7209 Am/24h (KT-12%)

Considering the amount of truck of flows 4 and 5 it is calculated that on the new bridge amount of trucks can be 12%.

Considering the tendency of increase of traffic intensity in our country it is assumed that during the next 20 years the average increase of traffic can be 3% per year. Therefore:

GVDI2027., for the new bridge = 7209 x (1+0.03 x 22) = 11 966 Am/dnn

### Calculation of coverage of road

Construction of coverage for the new bridge over the Venta river is calculated by using calculation programme ‘’SEGA 3’’, and checked after manual ‘’Designing of non-torpid coverage of road (RTU 1997.).

Calculation of average daily load of the carriageway in 1 year of service

Nb,1 = VDIsmA, 2005 x Ss,vid  (NAS100kN/24h)

Defining load of the carriageway Nb,1 or reduced intensity of heavy traffic during the first year of service there is considered information worked out by SIA „Solvers” on traffic intensity and composition of heavy traffic. If the year of opening of traffic is considered year 2009, then,

VDIsmA, 2009g = VDIsm, bz (1 + ps)ts = 865 x (1+0.03 x 4) = 969 SmA/24h

VDIsm, bz – intensity of trucks in the year of survey – year 2005;

ps – annual increase of load between year of survey and the first year of serving - (2009);

ts – time from year of prognosis base till the first year of serving ;

Average coefficient of summary impact Ss,vid is acquired by analysing composition of heavy transport.  Defining Ss,vid, heavy vehicles are divided in three groups:

|  | Division in groups of heavy vehicles |  |  |
| --- | --- | --- | --- |
| SmA length (m) | 5-8.5 | 8.5-17 | >17 |
| SmA proportion Ism = 12% | 4% | 5% | 3% |
| Impact coefficient Ss,gr | 0.135 | 1.58 | 1.264 |
| Ss,vid = (4 x 0.135 + 5 x 1.58 + 3 x 1.264) /12 = 1.02 |  |  |  |

Therefore,

Nb,1 = 969 x 1.02 = 988 (NAS100kN/24h)

NAS100kN – Nominal load on axle - 100kN;

Average daily load of the carriageway in the first year of serving, evaluating coefficient of load on axle 11.5T fsm= 1.5

Nb,1 = VDIsm x Ss,vid x fsm  = 969 x 1.02 x 1.5 = 1482 (NAS100kN/24h)

Calculation of accumulated load on traffic line WjT

For defining length of expected serving period of coverage the coverage of roads is designed according load of the traffic line Wj,T.

The accumulated load of the traffic line during the serving period of the coverage of the traffic line is calculated 100kN and 80kN for the equivalent load on axle. 100kN equivalent load on axle use in calculations in which are used method of VSN 46-83, but in calculations of 80kN load on axle (ESAL) in which is used method of  AASTHO (for example, computer programme ‘’Sega3’’). The result is acquired by multiplying expression with load transition coefficient - (100/80)4.

Wj,T 100kN load on axle = 365 x Nb,1 x f1 x Ka = 7 267 495 (NAS100kN/T for years)

vai

Wj,T 80kN load on axle = 365 x Nb,1 x f1 x Ka x (100/80)4 = 17 732 690 (ESAL80kN/T for years)

where,

365 – number of days per year;

Nb,1 – load of the carriageway in the first year of serving (in both directions of traffic);

f1 – coefficient of calculation traffic lines (the most loaded); f1 = 0.5;

T – load calculation period;

Kat – coefficient of accumulation period; Ka = ((1 + p)T – 1)/p

p – annual coefficient of increase of heavy traffic p=0.03;

T – the expected length of serving period of the coverage in years.

Determination of the necessary elasticity module Evaj and the necessary structural number SNvaj

1) Determination of the necessary elasticity module Evaj after method of  VSN 46-83 (p.3.9.)

Evaj = 355 – 13 (10 – Wj,T) – (10 – Wj,T)10/(82 x 106) = 319.48 MPa

The necessary elasticity module can be defined also by formula p.3.10a

Evaj = 67 log(NjT) +59 = 268.48 MPa

where,

NjT – impact intensity on standardized load on axle (NAS) on the calculation traffic lines during the last year of serving (2029.g.)

NjT = Fj x Kp x Nb1 = 0.5 x 1.806 x 1482= 1338 NAS/20 in years

where,

Fj – coefficient of traffic line;

Kp – coefficient of increase of heavy traffic in 20 years;

Assumed quantity

Evaj = 319.48 MPa

2) Determination of the necessary structural number SNvaj of coverage after method of AASTHO by using computer programme  „SEGA-3”

|  | ### Characteristics, materials and requirements of strength | Design Input data and results |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A | Characteristics of traffic |  |  |  |  |
| 1. | Road category– A4 (state road of 1.category) |  |  |  |  |
| 2. | Calculation load on axle of coverage – Pa = 11.5t (115kN) |  |  |  |  |
| 3. | Base year of traffic prognoses |  | - | 2007 | [year] |
| 4. | Year of traffic opening (1st working year) |  | - | 2009 | [year] |
| 5. | Load of the carriageway in the first year | Nb1 | = | 1482 | [NAS / 24 h] |
| 6. | Annual increase of load | p | = | 0.03 | [-] |
| 7. | Calculation period of load | T | = | 20 | [years] |
| 8. | Average daily load of the traffic line during the last year | NjT | = | 1338 | [NAS / 24 h] |
| 9. | Load coefficient of the calculation traffic line | Fj | = | 0.50 | [-] |
| 10. | Accumulated load of the traffic line during the first year (year 2009) | Wj1 | = | 659935 | [ESAL / in the first year] |
| 11. | Accumulated load of the traffic line in T=20 years | WjT | = | 17732690 | [ESAL /T in years] |
| NAS means 100kN of equivalent load on axle, ESAL – 80KN of equivalent load on axle |  |  |  |  |  |
| B | ###### requirements of strength and safety |  |  |  |  |
| 1. | The instructed level of safety | R | = | 95 | [%] |
| 2. | General standard deviation | S0 | = | 0.45 | [-] |
| 3. | The designed serving ability of the coverage (p0 –pt) | dPSI | = | 2.00 | [-] |
| 4. | Structural number, the necessary | SNvaj | = | 6.09 | [-] |
| C | Characteristics of soil and district |  |  |  |  |
| 1. | Elasticity module of soil | Egr | = | 35 | Mpa |
| 2. | Kind of district | Vtip | = | 2. | [-] |

Therefore in further designing of the coverage it is necessary to evaluate the calculated necessary output dimensions Evaj = 319.48 MPa, and SNvaj = 6.09

Solution of coverage construction

The calculated construction of coverage is used at both accesses of the bridge.

After geological data in upper layers of the section to be constructed deposits sandy loam and clay (see drawings in the geological report).

Calculation of equivalent elasticity module Eekv after method VSN 46-83

For calculation of the equivalent elasticity module it is necessary to evaluate some more output dimensions

Kdr – level of road coverage safety = 0.95

Kst – coefficient of strength = 1.0

| 1 layer 2 layer 3 layer 4 layer 5 layer 6 layer | Hot bituminous concrete upper layer (SMA11 – according „Requirements of road specification of year 2005”) h = 4 cm, E = 3200 Mpa; Hot bituminous concrete sub layer (ACb22 – according „Requirements of road specification of year 2005”) h = 7 cm, E = 2400 Mpa; Hot bituminous concrete sub layer (ACb32 – according „Requirements of road specification of year 2005”) h = 10 cm, E = 1800 Mpa; Mixture of dolomite shivers (0-56mm – according „Requirements of road specification of year 2005”) h = 30 cm, E = 225 Mpa; Frost sustainable layer from sand of medium coarse h = 50 cm, E = 90 Mpa; Existing ground– clay, sandy loam E = 35 Mpa. |  |
| --- | --- | --- |

| Layer | E, (Mpa) | h, (cm) | h/Dd | E1/E2 | Eekv/E1 | Eekv (Mpa) |
| --- | --- | --- | --- | --- | --- | --- |
| 6 | 35 |  |  |  |  |  |
| 5 | 90 | 50 | 1,3514 | 0,3889 | 0,742 | 66,780 |
| 4 | 225 | 30 | 0,8108 | 0,2968 | 0,563 | 126,675 |
| 3 | 1800 | 10 | 0,2703 | 0,0704 | 0,117 | 210,600 |
| 2 | 2400 | 7 | 0,1892 | 0,0878 | 0,117 | 280,800 |
| 1 | 3200 | 4 | 0,1081 | 0,0878 | 0,101 | 323,200 |

Total equivalent elasticity module of coverage Eekv = 323.200 Mpa

#### 1. results of calculation (VSN 46-83 method)

##### Eekv/Evaj > Kst

1.0117 (323.20 / 319.480) > 1.00 (condition of elastic curving fulfils)

Allowable curving of coverage del = 1.3

Allowable curving of coverage in impact of dynamic load del,d = 0.63

(1.3 > 0.63) del > del,d (condition fulfils)

#### 2. Results of calculation (method AASTHO) for the type of coverage nr.1

| D | Characteristics of work ability of coverage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Structural number, calculated |  |  | SNapr |  |  | = | 6.11 |  |  |  | [-] |  |  |  |
| 2. | Work (service) period of the coverage if Vtip= 1 |  |  | Tklp |  |  | = | 12.9 |  |  |  | [years] |  |  |  |
| 3. | Accumulated load of the traffic line with load 8T |  |  | Wta |  |  | = | 10.2 milj. |  |  |  | ESAL/13 years |  |  |  |
| 4. | Accumulated load of the traffic line with load 10T |  |  | Nta |  |  | = | 4.18 milj. |  |  |  | NAS /13 years |  |  |  |
| E | Layer materials of the coverage, coefficients, thickness and drainage coefficients |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. | Hot bituminous concrete upper layer from SMA11 | a1 | = |  | 0.440 | D2 |  |  | = | 4 | cm |  |  |  |  |
| 2. | Hot bituminous concrete sub layer from ACb22 | a2 | = |  | 0.290 | D2 |  |  | = | 7 | cm |  |  |  |  |
| 3. | Hot bituminous concrete upper layer from ACb32 | a3 | = |  | 0.260 | D3 |  |  | = | 10 | cm |  |  |  |  |
| 4. | Mixture of dolomite shivers 0-56mm | a4 | = |  | 0.140 | D3 |  |  | = | 30 | cm |  | m4 | = | 1.1 |
| 5. | Draining sand of medium coarse | a5 | = |  | 0.090 | D4 |  |  | = | 50 | cm |  | m5 | = | 1.0 |
| Explanation: Coefficients of layers of coverage– (a) assumed according the elasticity module of the materials. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

SNapr > SNvaj (6.11 > 6.09) (condition fulfils)

After calculation it is seen that at the same time methods AASTHO and VSN guarantee that thickness of the selected construction of coverage ensures stability and durability of it.

Author:	engineer of „Projekts 3”

U. Pūcītis

Calculations made by engineer of „Projekts 3”

I. Gorda

Appendices

List of points to be outlined

| Top of road route, circumference and transition curves |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number | Elements of road route | Picket | Coordinates |  |
|  |  |  | X(N) North (m) | Y(E) East (m) |
| 1 | TS | 0+00.00 | 362442.67 | 355317.60 |
| 4 | RLS | 0+95.46 | 362470.23 | 355408.99 |
| 3 | VIRS-1 | 2+71.80 | 362521.15 | 355577.82 |
| 5 | RLB | 4+34.52 | 362666.72 | 355677.35 |
| 6 | PLS | 9+78.29 | 363115.59 | 355984.27 |
| 7 | RLS | 9+93.29 | 363128.45 | 355991.94 |
| 8 | VIRS-2 | 10+18.26 | 363156.77 | 356012.43 |
| 9 | RLB | 10+43.22 | 363174.16 | 355982.10 |
| 10 | PLB | 10+58.22 | 363182.74 | 355969.83 |
| 11 | TB | 10+70.00 | 363188.87 | 355959.77 |

Legend:

TS – beginning of route

PLS – beginning of the transition curve

RLS – Beginning of the circumference

VIRS – Top

PLB – End of the transition curve

RLB – End of the circumference

TB – End of the route

|  | Picket | Coordinates |  |  |
| --- | --- | --- | --- | --- |
|  |  | X(N) North (m) | Y(E) East (m) |  |
|  | 0+00.00 | 362442.67 | 355317.60 |  |
|  | 2+00.00 | 362510.62 | 355505.21 |  |
|  | 2+10.00 | 362515.52 | 355513.92 |  |
|  | 2+20.00 | 362520.60 | 355522.54 |  |
|  | 2+30.00 | 362525.84 | 355531.06 |  |
|  | 2+40.00 | 362531.25 | 355539.46 |  |
|  | 2+50.00 | 362536.83 | 355547.76 |  |
|  | 2+60.00 | 362542.58 | 355555.95 |  |
|  | 2+70.00 | 362548.49 | 355564.01 |  |
|  | 2+80.00 | 362554.55 | 355571.96 |  |
|  | 2+90.00 | 362560.78 | 355579.79 |  |
|  | 3+00.00 | 362567.16 | 355587.49 |  |
|  | 3+10.00 | 362573.69 | 355595.06 |  |

|  | Picket | Coordinates |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | X(N) North (m) | Y(E) East (m) |  |  |
|  | 3+20.00 | 362580.38 | 355602.50 |  |  |
|  | 3+30.00 | 362587.21 | 355609.80 |  |  |
|  | 3+40.00 | 362594.18 | 355616.96 |  |  |
|  | 3+50.00 | 362601.30 | 355623.99 |  |  |
|  | 3+60.00 | 362608.56 | 355630.87 |  |  |
|  | 3+70.00 | 362615.95 | 355637.60 |  |  |
|  | 3+80.00 | 362623.48 | 355644.19 |  |  |
|  | 3+90.00 | 362631.13 | 355650.62 |  |  |
|  | 4+00.00 | 362638.91 | 355656.90 |  |  |
|  | 4+10.00 | 362646.82 | 355663.02 |  |  |
|  | 4+20.00 | 362654.85 | 355668.98 |  |  |
|  | 4+30.00 | 362663.00 | 355674.78 |  |  |
|  | 4+40.00 | 362671.24 | 355680.44 |  |  |
|  | 4+50.00 | 362679.49 | 355686.09 |  |  |
|  | 4+60.00 | 362687.75 | 355691.73 |  |  |
|  | 4+70.00 | 362696.00 | 355697.38 |  |  |
|  | 4+80.00 | 362704.26 | 355703.02 |  |  |
|  | 4+90.00 | 362712.51 | 355708.67 |  |  |
|  | 5+00.00 | 362720.77 | 355714.31 |  |  |
|  | 5+10.00 | 362729.02 | 355719.95 |  |  |
|  | 5+20.00 | 362737.28 | 355725.60 |  |  |
|  | 5+30.00 | 362745.53 | 355731.24 |  |  |
|  | 5+40.00 | 362753.79 | 355736.89 |  |  |
|  | 5+50.00 | 362762.04 | 355742.53 |  |  |
|  | 5+60.00 | 362770.30 | 355748.18 |  |  |
|  | 5+70.00 | 362778.55 | 355753.82 |  |  |
|  | 5+80.00 | 362786.81 | 355759.46 |  |  |
|  | 5+90.00 | 362795.06 | 355765.11 |  |  |
|  | 6+00.00 | 362803.32 | 355770.75 |  |  |
|  | 6+10.00 | 362811.57 | 355776.40 |  |  |
|  | 6+20.00 | 362819.83 | 355782.04 |  |  |
|  | 6+30.00 | 362828.08 | 355787.69 |  |  |
|  | 6+40.00 | 362836.34 | 355793.33 |  |  |
|  | 6+50.00 | 362844.59 | 355798.97 |  |  |
|  | 6+60.00 | 362852.84 | 355804.62 |  |  |
|  | 6+70.00 | 362861.10 | 355810.26 |  |  |
|  | 6+80.00 | 362869.35 | 355815.91 |  |  |
|  | 6+90.00 | 362877.61 | 355821.55 |  |  |

|  | Picket | Coordinates |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | X(N) North (m) | Y(E) East (m) |  |  |
|  | 7+00.00 | 362885.86 | 355827.20 |  |  |
|  | 7+10.00 | 362894.12 | 355832.84 |  |  |
|  | 7+20.00 | 362902.37 | 355838.48 |  |  |
|  | 7+30.00 | 362910.63 | 355844.13 |  |  |
|  | 7+40.00 | 362918.88 | 355849.77 |  |  |
|  | 7+50.00 | 362927.14 | 355855.42 |  |  |
|  | 7+60.00 | 362935.39 | 355861.06 |  |  |
|  | 7+70.00 | 362943.65 | 355866.71 |  |  |
|  | 7+80.00 | 362951.90 | 355872.35 |  |  |
|  | 7+90.00 | 362960.16 | 355877.99 |  |  |
|  | 8+00.00 | 362968.41 | 355883.64 |  |  |
|  | 8+10.00 | 362976.67 | 355889.28 |  |  |
|  | 8+20.00 | 362984.92 | 355894.93 |  |  |
|  | 8+30.00 | 362993.18 | 355900.57 |  |  |
|  | 8+40.00 | 363001.43 | 355906.22 |  |  |
|  | 8+50.00 | 363009.69 | 355911.86 |  |  |
|  | 8+60.00 | 363017.94 | 355917.51 |  |  |
|  | 8+70.00 | 363026.20 | 355923.15 |  |  |
|  | 8+80.00 | 363034.45 | 355928.79 |  |  |
|  | 8+90.00 | 363042.71 | 355934.44 |  |  |
|  | 9+00.00 | 363050.96 | 355940.08 |  |  |
|  | 9+10.00 | 363059.21 | 355945.73 |  |  |
|  | 9+20.00 | 363067.47 | 355951.37 |  |  |
|  | 9+30.00 | 363075.72 | 355957.02 |  |  |
|  | 9+40.00 | 363083.98 | 355962.66 |  |  |
|  | 9+50.00 | 363092.23 | 355968.30 |  |  |
|  | 9+60.00 | 363100.49 | 355973.95 |  |  |
|  | 9+70.00 | 363108.74 | 355979.59 |  |  |
|  | 9+80.00 | 363117.00 | 355985.24 |  |  |
|  | 9+90.00 | 363125.49 | 355990.50 |  |  |
|  | 10+00.00 | 363134.80 | 355994.10 |  |  |
|  | 10+10.00 | 363144.70 | 355995.29 |  |  |
|  | 10+20.00 | 363154.59 | 355993.99 |  |  |
|  | 10+30.00 | 363163.85 | 355990.28 |  |  |
|  | 10+40.00 | 363171.91 | 355984.40 |  |  |
|  | 10+50.00 | 363178.33 | 355976.76 |  |  |
|  | 10+60.00 | 363183.66 | 355968.31 |  |  |
|  | 10+70.00 | 363188.87 | 355959.77 |  |  |

Summary of work amount

| Number | Kind of work | Unity | Total amount expected in the project | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Dismantling works |  |  |  |
| 1.1 | Dismantling of coverage |  |  |  |
|  | - bituminous concrete | m2 | 2475 |  |
|  | - Coverage of cement concrete | m2 | 500.5 |  |
|  | - Coverage of grant | m2 | 18566 |  |
| 1.2 | Dismantling of constructions |  |  |  |
|  | - dismantling of concrete boards | m | 85.8 |  |
|  | - dismantling of existing metal fence | m | 93.5 |  |
| 2 | Preparatory works |  |  |  |
| 2.1 | Outline of route | gab. | 93 |  |
| 2.2 | Outline of separate points | gab. | 85 |  |
| 2.3 | Cut down of trees, uproot of roots, transportation | gab. | 35 |  |
| 2.6 | Liquidation of bushes | ha. | 0.2 |  |
| 2.7 | Excavation of ditches | m3 | 920 |  |
| 2.8 | Fastening of ditch channel with MACADAMAT type of anti erosion carpet | m2 | 28 |  |
| 2.9 | Fastening of ditch channel with mixture of dolomite shivers, h=20 | m2 | 23 |  |
| 3 | Land works |  |  |  |
| 3.1 | Establishment of bottom for construction of coverage | m2 | 20528 |  |
| 3.2 | Amount of ground to be transported | m3 | 4790 |  |
| 3.3 | Ground of embankment | m3 | 23614 |  |
| 4 | Construction of coverage |  |  |  |
| 4.1 | Carriageway of the basic road |  |  |  |
|  | - frost sustainable layer, h=50cm | m3 | 10320 |  |
|  | - Mixture of dolomite shivers, h=30cm | m2 | 6095 |  |
|  | - sub layer of hot bituminous concrete ACb32, h=10cm | m2 | 5121 |  |
|  | - sub layer of hot bituminous concrete ACb22, h=7cm | m2 | 5121 |  |
|  | - upper layer of hot bituminous concrete SMA11, h=4cm | m2 | 5121 |  |
|  | - Priming of hot bituminous concrete | m2 | 15362 |  |
| 4.2 | Carriageway of the access |  |  |  |
|  | - Coverage of grant, h=18cm | m2 | 1523 |  |
|  | - Frost sustainable layer, h=30cm | m2 | 574 |  |
| 4.3 | Sidewalks |  |  |  |
|  | - Frost sustainable layer, h=30cm | m3 | 39 | Amount is given for sidewalks in Fabrikas street (along the buildings towards the garage). For the other – amount is given at the basic road. |
|  | - foundation of shiver, h=12cm | m2 | 2195 |  |
|  | - offal of shiver or sand stratum šķembu, h=3cm | m2 | 2195 |  |
|  | - concrete pavement ,h=6cm | m2 | 2195 |  |

| Number | Kind of work |  | Unity |  | Total amount expected in the project |  | Remarks |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.4 | Boulder pavement in the cement tartar |  |  |  |  |  |  |  |  |
|  | - construction of foundations of shiver, h=20cm |  | m2 |  | 9 |  |  |  |  |
|  | - boulder pavement |  | m2 |  | 9 |  | For strengthening of gully |  |  |
| 5 | Culverts and constructions |  |  |  |  |  |  |  |  |
| 5.1 | Construction of culverts |  |  |  |  |  |  |  |  |
|  | - Ø800 PE, class of deposit T=8 |  | m |  | 19 |  |  |  |  |
|  | - Ø1000 PE, class of deposit T=8 |  | m |  | 45 |  |  |  |  |
|  | - excavation of trenches |  | m3 |  | 230 |  |  |  |  |
|  | - foundation of shiver, h=20cm |  | m2 |  | 157 |  |  |  |  |
|  | - filling of trench with ground, which contains lesser than 5% of clay particles |  | m3 |  | 197 |  |  |  |  |
| 5.2 | Renovation zone of concrete pavement |  |  |  |  |  |  |  |  |
|  | - concrete pavement |  | m2 |  | 3 |  |  |  |  |
|  | - shiver offal |  | m2 |  | 3 |  |  |  |  |
| 5.3 | Concrete boards |  |  |  |  |  |  |  |  |
|  | - oblique |  | m |  | 10 |  |  |  |  |
|  | - BR100.20.8 |  | m |  | 2155 |  |  |  |  |
|  | - BR100.22.15 |  | m |  | 20 |  |  |  |  |
|  | - BR100.30.15 |  | m |  | 1360 |  |  |  |  |
| 6 | Equipment |  |  |  |  |  |  |  |  |
|  | - Mounting of road sign support |  | piece |  | 10 |  |  |  |  |
|  | - mounting of road sign Nr. 103 |  | piece |  | 1 |  |  |  |  |
|  | - mounting of road sign Nr. 104 |  | piece |  | 1 |  |  |  |  |
|  | - mounting of road sign Nr. 206 |  | piece |  | 3 |  |  |  |  |
|  | - mounting of road sign Nr. 326 |  | piece |  | 3 |  |  |  |  |
|  | - mounting of road sign Nr. 708 |  | piece |  | 2 |  |  |  |  |
|  | - mounting of pedestrian safety barriers |  | m |  | 1024 |  |  |  |  |
|  | - protective pipes of communications DVK 110 |  | m |  | 785 |  |  |  |  |
| 7 | Greening |  |  |  |  |  |  |  |  |
|  | - gliding of land slopes and territory, greening with land, h=10cm |  | m2 |  | 17997 |  |  |  |  |
|  | - sowing of perennial lawn |  | m2 |  | 17997 |  |  |  |  |
| Remarks: |  |  |  |  |  |  |  |  |  |
| 1 | For those kind of works for which is indicated measure of capacity, capacity for the materials is compact. |  |  |  |  |  |  |  |  |
| 2 | The contractor must evaluate all the necessary materials and additional works mentioned in the „Summary of work amount” which are not mentioned in this list, but without what will not be able execution in full amount in technologically correct way and according the norms being in force |  |  |  |  |  |  |  |  |

Contents of drawings

### GT – Road part

- General data 	GT-0

- Layout  M 1:250	GT-1-1

- Layout M 1:250	GT-1-2

- Layout M 1:250	GT-1-3

- Layout M 1:250	GT-1-4

- Layout M 1:250	GT-1-5

- Horizontal and vertical planning M 1:250	GT-2-1

- Horizontal and vertical planning M 1:250	GT-2-2

- Horizontal and vertical planning M 1:250	GT-2-3

- Horizontal and vertical planning M 1:250	GT-2-4

- Horizontal and vertical planning M 1:250	GT-2-5

- Profile lengthwise 	GT-3

- Sections A-A	GT-4-1

- Sections B-B, C-C, D-D	GT-4-2

- Construction of culverts Ø800 and Ø1000 	GT-5