Torņu iela 4-IIIC, 203.kab., Riga, LV 1050, Latvia, phone +371 7223144, fax +371 7223830, e-mail: witbo@apollo.lv

Reg. Nr. 40003353498, Account No. LV45UNLA0050001567270

Latvijas Unibanka, Ridzene affiliate/bank code SWIFT UNLALV 2X

The Client:	Ventspils Free Port Authority

Jāņa iela 19, Ventspils, LV 3601

Project Nr.:	Let89-3

Name of the Project:                             TECHNICAL PROJECT OF NEW BRIDGE

AND ACCESS ROADS AT VENTSPILS

Stage of construction project:	Technical project

Volume:	Volume 1.2

General part

Manager of construction Project:	O. Zivtiņš

Author of construction project:	SIA ”Witteveen+Bos Latvia”

2007

Composition of project

Technical project of new bridge and access roads at Ventspils

Volume 1 .1	General part (In Latvian)

GT – road part

ŪKT – part of water supply and sewerage external network

ELT – part of electrical supply and communication external netwotrk

Volume 1.2	General Part

Volume 2 		BK – building structures

Volume 3		summary of main works

Contents

Composition of project 2 Contents 3 General part 4 Ministry of Transport of Latvia technical regulations copy 5 A/S „Latvenrego” west region technical regulations copy 6 VAS „Latvijas Dzelzceļš” technical regulations copy 7 P.I. „Ventspils Komunālā pārvalde” technical regulations 10 SIA of Municipality „Ventspils siltums” technical regulations copy 11 SIA „SkatVis” technical regulations copy 12 SIA „Latvijas Propāna Gāze” technical regulations copy 13 SJSC „State Roads of Latvia” technical regulations copy 14 VUGD Ventspils team technical regulations copy 15 VA „Sabiedrības veselības aģentūra” Ventspils affiliate technical regulations copy 16 Municipality Ltd. „ŪDEKA” technical regulations copy 17 Ventspils city Municipality Ltd. „Pārventas siltums” technical regulations copy 18 SIA „LATTELECOM” technical regulations copy 19 State environmental service, Ventspils regional environmental authority technical regulations copy 20 Ventspils City Council Order copy 22 Ventspils City Council planning and arcitectural task nr.34 copy 23 Ventspils City Council planning and arcitectural task nr.304 copy 28

GT, ŪKT and ELT part explanatory notes, main works, specifications and drawings in VOLUME 1.1 .

General part

Ministry of Transport of Latvia technical regulations copy

Translation from Latvian

Ministry of Transport of Latvia

Maritime Administration of Latvia

Registration number: 40003022705

Trijādības street 5, Riga, LV-1048; phone: +371 7062101; fax: + 371 7860082;

e-mail: lja@lja.bkc.lv

Date: May 11, 2007

Nr. 1-14/209

Riga

Reply to the letter dated on 14th April, 2007; No. 1-5.1/716

To: Ventspils Free Port Authority

On technical regulations

SJSC „Maritime administration of Latvia” reviewed the submitted terms of reference for the „Development of construction Technical project of the new bridge over Venta river, Ventspils” and coordinate it with the following conditions:

- To equip the spans and pilons of the shipping zone of the bridge with the navigation marks and lights according requirements of paragraph IX of regulation No.158 of Cabinet of Ministers (issued on March1, 2005).

- The Technical project must be coordinated with the captain of Ventspils Free Port and submitted for the acceptance to the Maritime Administration of Latvia.

Deputy director 								A.Brokovskis

R.Čate

7062104

A/S „Latvenrego” west region technical regulations copy

Translation from Latvian

LATVENERGO

Distribution network

Stock Company Latvenergo

Distribution network

WESTERN REGION

United registration No. 40003032949

Riga street 56, Liepāja, LV – 3401, Latvia

Phone: +371 7728777; Fax: + 371 3423105

klientu.serviss@latvenergo.lv

www.latvenergo.lv

Liepāja

April 24, 2007; No. 30R4E0-01/615

Reply to the letter dated on April 13, 2007; no. 3-7.2.1/698

To: VFPA

Jāņa street 19, Ventspils,

LV – 3601, Fax: 36 21297

On technical regulations

„Construction of new transport bridge at Ventspils”

- To expect replacement of sections F-11, F-19, F-14 of cables 10kV in case if due to reclamation they stay deeper than 1,0m or in the carriageway (brand of the cables ASB-10 3x240).

- To expect cable trough of 5 canal electrical cables with pipes of diameter not lesser than 160mm.

- The Technical project must be coordinated with SC Latvenergo, Distribution Network development department of Western region and with SC Latvenergo, Distribution Network Exploitation department of Western region, Ventspils department.

- Technical regulations are valid for two years.

Head of exploitation department						Kristaps Kerve

Rolands Agafonovs

63610972

VAS  „Latvijas Dzelzceļš” technical regulations copy

Translation from Latvian

SJSC Latvian railroads

Registration No. 40003032065

Authority of Infrastructure

Gogoļa street 3, Riga-50, LV – 1547. Phone: 7234404, 7234405: Fax: 7233838

Riga

Date: April 18, 2007

Nr. D1-4/263

Reply to the letter dated on April 13, 2007; No. 1-5.1/695

To: manager of VFPA

Mr. I.Sarmulis

Jāņa iela 19, Ventspils, LV – 3601

On technical regulations

Validity term of technical regulations Nr.21 issued on March 17, 2005 on construction of new bridge over the Venta river and reconstruction of the existing bridge is prolonged till September 1, 2008.

Deputy Director of department of infrastructure 					M.Riekstiņš

Goldfarbs

7234732

Translation from Latvian

APPROVE:

SJSC “Latvian railroads”

Director of Department of infrastructure

M.Jagodkins

25th March, 2005

Technical regulations of

Construction of new traffic bridge over Venta river

and reconstruction of the existing traffic bridge, Ventspils

March 17, 2005									No.21

- Reconstruction and construction of the bridge must be carried out by observing Construction regulations of LR, Construction regulations of railroads, standard as of September 7, 2000 “Size of approximation and of rolling stocks of railroad constructions” LVS 282 and requirements of technical norms being in force.

- In the project there must be indicated the existing railroad communications (electric supply and communication cables, overhead transmission line); location can be clarified at Jelgava signalisation and communication division. During work execution there must be ensured maintaining of railroad constructions, equipment and communications. In case of necessity from the construction zone to expect replacement of the railroad communications on means of the Client.

- The bridge to be designed must be located behind borders of railroad land distribution zone.

- During the design of pilons of bridge, there must be observed standard LVS 282 and protective zone of railroad communications.

- Pilons of the new bridge must be located in one span with the existing pilons of the railroad bridge.

- Pass of ice must be ensured between traffic and railroad bridges.

- There must be ensured drainage of water from the bridge behind the borders of the railroad.

- To expect strengthening of coastal pilons of the constructions of railroad bridge.

- To expect strengthening of the coasts against erosion before the bridges from the side of the upper course of the river.

- The new traffic bridge must be covered with shields of height 1,5 – 2 m for the reason that during the strong winds there must be excluded blowing of anti-icing reagents from the carriageway of the traffic bridge to the metal constructions of the railroad bridge.

- It is not allowed execution of immersion of piles by use of vibration closer than 15 metres from the constructions of railroad bridge.

- In the “C “ zone of approximation size and above this zone the works must be carried out in “windows” which is formatted in the interval of the railroad schedule.

- There must be ensured maintaining of the work technologies of the access roads of the railroad (in the work execution zone of bridge construction and reconstruction) by means of passing and removing the coaches.

- To expect establishment of cable trough for the insertion of railroad cable-communications on route: “Communication houses” at the building of passenger station of the railroad at Fabrika street – along Fabrika street till the railroad derivation to the direction of Ventspils Southern station – along sewerage of the new traffic bridge – till Rūpniecības street – along Rūpniecības street till the existing traffic bridge – over the bridge to be reconstructed – till the cable well at the dead end of Ventspils passenger station. After completion of the construction works the established sewerage must be handed over to the Jelgava signalisation and communication division by making the deed. Copy of the deed must be sent to the Telecommunication department of Authority of Infrastructure.

- During work execution question regarding train traffic schedule must be coordinated with the Ventspils station.

- During work execution there must be expected protection against different construction material and other subject fall from height from the bridge to the railroad bridge as well as protection against contamination of railroads with the debris.

- After completion of works to clean up the territory, and with the ballast cleaning machine to clean the ballast layer of the railroad along 50m from the bridge axle on each side.

- Complete documentation must be coordinated with Jelgava road division, jelgava signalisation and communication division, Ventspils railroad station and project coordination commission of SJSC “Latvian railroads”.

- To resolve the ambiguity in the future development of railroad infrastructure according the developed “Actualisation of development scheme of Ventspils railroad networks” the project must be coordinated with SIA “Transceltnieks”.

- One copy of each project document must be submitted to the Jelgava road division and Jelgava signalisation and communication division.

- To expect payment for execution of supervision to the representatives of the railroad.

These technical regulations are valid one year.

Head of Technical part of Authority of Infrastructure 			A.Konstantinovs

Remark: Technical regulations are issued to deputy manager of VFPA Mr. A.Mazalis, on 23 February, 2005, letter No. 1-7.2.2/273-11, phone 3602318, fax: 3621297

Goldfarbs

7234732

P.I. „Ventspils Komunālā pārvalde” technical regulations

Translation from Latvian

Republic of Latvia

Establishment of Ventspils Municipality

“Komunālā pārvalde”

Užavas street 8, Ventspils, LV – 3600, phone: 3624269; fax: 3626379; e-mail: kom.parvalde@ventspils.gov.lv

Ventspils

14 April, 2007; No. 1-26/704

To: Manager of VFPA

Mr. I.Sarmulis

On new bridge and bridge over Venta river, Ventspils

TECHNICAL REGULATIONS

Establishment of Ventspils municipality “Komunālā pārvalde” for the projects of bridge over the Venta river and construction of new bridge at the railroad bridge requests to evaluate and to prepare the following requirements:

- The daily intensity of the existing bridge after the data of May, 2005 is approximately 31 000 units per day. During designing please take into account this intensity and increase of traffic intensity for the at least next 30 years.

- Considering the existing traffic intensity in the project of work organisation and contents of project (amount) to expect equipping of traffic organisation and of access roads with corresponding road – signs and limitations of traffic flow. To work out traffic schemes for different stages of the bridge reconstruction when there will be closed one or another bridge.

- For the parapets of the bridge, road signs, lighting supports use zincified construction elements which later will reduce the exploitation costs.

- For the lanterns use Na lighting or another which ensures yellow light and is economical.

- For the pedestrian zone to expect colourful cable stone coverage.

- For the carriageway to expect construction of asphalt, the component must be proposed in the sketch project.

- Thermo stitches in the spans and its constructions must be proposed in the sketch project.

- For the marking use thermoplast.

Director 								A.Kausenieks

G.Bendrats

3622515

Translation from Latvian

Republic of Latvia

SIA of Municipality „Ventspils siltums” technical regulations copy

SIA of Municipality “Ventspils Siltums”

United registration number 40003007655

Brīvības street 38, Ventspils, LV – 3601, Phone, fax: 3627902; e-mail: vent.siltums@ventspils.gov.lv

Ventspils

Date: March 8, 2005							Nr. 8-1.1/350

Reply to the letter dated on February 23, 2005; Nr. 1-7.2.1/273-2

To: VFPA

Deputy Manager Mr. A.Mazalis

Jāņa street 19, Ventspils, LV – 3601

On technical regulations

According regulation No.41 of Cabinet of Ministers as of 28 February, 1995 “Regulations of supply and usage of heating energy” there is no need of technical regulations for the reconstruction of existing traffic bridge over the Venta river and for the construction of new traffic bridge.

Chairman of the board							O.Svētiņš

V.Blūms

3627903

SIA  „SkatVis” technical regulations copy

Translation from Latvian

SIA “SkatVis”

Registration number 120300274

Ģertrūdes iela 13, Ventspils, LV – 3600, phone: 3628851; fax: 3628852; e-mail: skatvis@skatvis.net

Ventspils

Date: March 9, 2005

No. 1-15/23

To: VFPA

On technical regulations

Replying to your letter No.1-7.2.1/273-6, dated on 23 February, 2005, we inform, that SIA “SkatVis” in the object “Reconstruction of existing traffic bridge over the Venta river and construction of new bridge beside the railroad bridge” does not have underground communications and does not expect to install them in the nearest future.

Member of the Board,

Technical director							S.Larčenko

SIA „Latvijas Propāna Gāze” technical regulations copy

Translation from Latvian

SIA “LPG”

Kurzemes prospect 19, Riga, United registration No. 40003493561

Ventspils gas filling station, Kustes dambis 33, Ventspils, LV – 3602

Phone: 3607168, 3664143

Ventspils

Date: March 14, 2005							Nr. 04-48

Reply to the letter dated on February 23, 2005, No. 1-7.2.1/273-5

To: Ventspils Free Port Authority

On technical regulations

In the object “Reconstruction of existing traffic bridge over the Venta river and construction of new bridge beside the railroad bridge” there are no gas communications.

Head of Ventspils gas filling station 					Arvis Rožkovs

I.Belijs

3664143

SJSC „State Roads of Latvia” technical regulations copy

Translation from Latvian

SJSC “State Roads of Latvia”

Technical department

Registration number 40003344207

Gogoļa street 3, Riga, LV – 1050; Phone: 7028169; fax: 7028171

www.lad.lv

Riga, March 17, 2005							Nr. 2.4-94

To: Deputy manager of VFPA

Mr. A.Mazalis

TECHNICAL REGULATIONS

For the work out of construction project of reconstruction of

existing traffic bridge over the Venta river and construction of

new bridge over the Venta river in Ventspils

SJSC “State roads of Latvia” consents for the construction designing of bridges over the Venta river at Ventspils with the following conditions:

- The Construction projects have to be worked out according requirements of Law on Construction and norms and regulations being in force of LR.

- The construction projects of bridges must be worked out by determining the calculation loads according LVS EN 1991-2, α = 1 for the new bridge, but for reconstruction α > 0,8.

- Size of carriageway of the bridge – according LVS 190-2:1999.

- Opening of the new bridge must be selected according of 1% security of maximal water level of the river.

- The cone slope at the accesses of the bridges must be protected at full height, but the side slopes of the access embankment – at the height of 0,5m above the highest calculated water level with 1% of security.

- Pedestrian sidewalks of the bridge must be connected with the sidewalks of the access roads in the most optimal way.

- Lighting on the bridge must be connected with the lighting of the accesses in the most optimal way.

- In the construction projects for the construction period there must be solved the traffic organisation.

- The construction project must be coordinated at the “Bridge department”of SJSC “State roads of Latvia”.

Head of the bridge department						M.Dūzelis

I.Jurka

7028130

VUGD Ventspils team technical regulations copy

Translation from Latvian

STATE FIRE FIGHTING AND RESCUE SERVICE (VUGD)

VENTSPILS TEAM

Registration number: 90000036168

Inženieru street 1, Ventspils, LV – 3601; fax: + 371 36-22026, phone: + 371 36-22389;

e-mail: vpils@vugd.gov.lv

Ventspils

29 March, 2005

Name of legal person: Free port of Ventspils Authority

Registration number of legal person: No. 90000284085

Address of legal person: Jāņa iela 19, Ventspils, LV – 3601

TECHNICAL REGULATIONS NO.59

For the construction designing

- Name of construction object and address: Reconstruction of existing traffic bridge over the Venta river and construction of new bridge beside the railroad bridge in the territory of Free port, Ventspils

- Kind of construction: reconstruction of existing bridge and onstruction of new bridge

- Initiator of construction: Ventspils Free Port Authority

- Submitter of documents and submitted materials: application No. 185 as of March 8, 2005 of VFPA; situation plan; clarification note; Terms of Reference.

- Requirements of fire security of laws and regulations:

  - for the placement of constructions, space of fire security and for drive way: according the requirements of LBN 201-96. Recommend to carry out the designing by using the requirements indicated in the LBN 100 and CN and N 2.05.03-84.

  - For the security of fire fighting, rescue and civil protection works: during the reconstruction works to provide movement of operative transport over the bridge. Bridge lifting times must be coordinated with the VUGD Ventspils team.

  - For the internal and external water supply for the needs of fire fighting: According LBN 221-98, LBN 222-96 and LBN 201-96

  - For the electrical supply of fire protection system: according the requirements of laws and regulations of establishment of electric facilities

  - For the lightning protection: according the requirements of LBN 201-96; paragraph 3.1.6

  - For the fire protection systems: according the requirements of LBN 201-96, paragraph 7.11

  - For the planning and constructive solutions: according the requirements of LBN 201-96 and LBN 203-97

  - Other suggestions: the project must be submitted for revision to VUGD Ventspils Team.

Payment category: complicated

Technical regulations are valid one year.

These technical regulations can be arbitrated in one month time starting the day of they become in force to the superior official: head of the VUGD, Maskavas street 5, Riga.

Prepared by: commander of Ventspil team G.Pavlovs

Received by: construction engineer A.Jēkabsons (30 March, 2005)

VA „Sabiedrības veselības aģentūra” Ventspils affiliate technical regulations copy

Translation from Latvian

State agency “Public Health agency”(SVA)

Ventspils affiliate

Hygienic task of designing

March 29, 2005								Nr. 67

- Name of object : reconstruction of existing traffic bridge over the Venta river and construction of new traffic bridge

- Address of object: Ventspils

- Owner of object: Free Port of Ventspils Authority

- Submitted documents: 1) application No. 1-7.2.1/273-8 submitted by deputy manager of VFPA, Mr. A.Mazalis

2) Terms of Reference of VFPA.

- Reviewing the submitted documents on March 9, 2005, registration No. 362, it is stated: in the project there will be expected reconstruction of existing traffic bridge over the Venta river and construction of new traffic bridge. The new bridge is expected to be beside the existing railroad bridge connecting Rūpniecības and Fabrikas streets. Length of the bridge – 168 metres, two driving zones – one fro each direction, and one pedestrian zone.

- Task: designing must be carried out according construction norms of LR being in force.

Head of the SVA Ventspils affiliate or the authorised representative

I.Vinogradova

Name of the executor: Linda Valtasa

Municipality Ltd. „ŪDEKA” technical regulations copy

Translation from Latvian

Municipality Ltd. “’UDEKA”

Talsu iela 65, Ventspils, phone; 3661495; fax: 36 61912

January 20, 2006

Nr. 05-03/73

TECHNICAL REGULATIONS

FOR DESIGNING OF WATER MAIN AND SEWERAGE SYSTEMS

THE CLIENT: Ventspils Free Port Authority

OBJECT: construction of new traffic bridge over the Venta river, Ventspils

- The object to be designed must be added to:

- the water pipe to be designed in the Jaunprojektējamā iela;

- the rain water sewage to be designed in the Jaunprojektējamā iela.

- Pressure in the water pipe system – 4,0 up to 5,0 bars.

- The Client must construct:

Water pipe

  - To construct the water pipe of the industrially insulated PE pipes. Diameter of the pipes is stated during designing and calculations.

  - The water pipe must be connected to the water pipe to be designed at Jaunprojektējamā street at the left and right banks of the river Venta.

Rain water sewage

  - To construct rain water sewage system with pipes from PVC or PEH material. Diameter of the pipes is stated during designing and calculations. In the certain places to construct check pits and sumps.

  - The rain water sewage system must be connected to the rain water sewage system to be designed at Jaunprojektējamā street at the left and right banks of the river Venta.

- Additional requirements:

  - To observe requirements of LBN 222-99, LBN 223-99 and LBN 100.

  - Project documentation must be coordinated with Ltd “Udeka”.

- The Technical regulations are valid two years.

- Technical regulations No. 05-03/505issued on April 28, 2005 are abolished.

Chairman of the board 								G.Krauze

A.Šlangens

36 07286

Ventspils city Municipality Ltd. „Pārventas siltums” technical regulations copy

Translation from Latvian

Republic of Latvia

Ventspils City Municipality Ltd.

“PĀRVENTAS SILTUMS”

REG. No. 41203007943

Talsu street 84, Ventspils, LV – 3602

Phone: 3602200; fax: 3602210; e-mail: parventas.siltums@ventspils.gov.lv

Ventspils

May 18, 2005								Nr. 1-5.2/253

Reply to the letter dated: February 23, 2005; No. 1-7.2.1/273-3

To: Ventspils Free Port Authority

ON TECHNICAL REGULATIONS

Ltd. “Pārventas siltums” declares, that in the object of VFPA “Reconstruction of existing traffic bridge over the Venta river and construction of new traffic bridge beside the railway bridge, Ventspils” they do not have their communications.

Chairman of the board							V.Reinbergs

Reinicane

3602220

SIA „LATTELECOM” technical regulations copy

Translation from Latvian

SIA LATTELECOM

Registration number 000305278

Vaļņu stret 30, Riga, LV – 1050

Phone 7055000; fax 7055481

e-mail: lattelecom@lattelecom.lv

TECHNICAL REGULATIONS NO 2995-6/147

Date: May 20, 2005

Substantiation: Order Nr. 3-7.21/273-12

Claimant: Ventspils Free Port Authority

Jāņa street 19, Ventspils, LV – 3601

Contact person: A.Jēkabsons; phone: 36 02318

Address of object: reconstruction of traffic bridge over the Venta river and construction of new traffic bridge beside the railway bridge at Ventspils

Reason for technical regulations: protection of communications of Lattelecom

DESCRIPTION OF TECHNICAL REGULATIONS

For implementation of the requested works there is necessary:

1. Description of necessary works	Beside the existing traffic bridge over the Venta river and in the river bed there is optical communication cable

For the protection of communication network (provision of uninterrupted operation of network): to maintain and protect the existing communications

If the communications hinders to carry out reconstruction of traffic bridge over the Venta river, to expect replacement of them, each case coordinating with Lattelecom and expecting financing of the works

2. Conditions of execution of TR	To observe technical regulations of construction of telecommunication network

- Annexed: schemes and/or photo of the work place

See: scheme of location of optical cable beside the traffic bridge and in the river bed of Venta river

Remarks: Network maintenance department of Ltd. Lattelecom Kurzeme region proposes to carry out designing, construction and mounting works in your objects. For information please call 36 24424, V.Prusakovs

For execution of the above mentioned works it is necessary to work out the project. Designing and construction works are to be implemented according technical standards of “Lattelecom”. In case of project work out, please coordinate it with:

- “Lattelecom” TDTUD KR TUN, address: Ventspils, Jūras street 9, 2nd floor, phone: 3624424

After completion of works, the documentation has to be handed over to:

- “Lattelecom” TDTUD KR TUN, address: Ventspils, Jūras street 9, 2nd floor, phone: 3624424

Technical regulations prepared by: 	V.Prusakovs

Date: 19th May, 2005

State environmental service, Ventspils regional environmental authority technical regulations copy

Translation from Latvian

TECHNICAL REGULATIONS No. 9.5-5.1/106

Valid till: 26th July, 2010

Are invalid if the activity is not started

in three year time after issue

State Environmental service, Ventspils regional environmental authority

Address: Dārzu street 2, Ventspils, LV – 3601

Phone: 3625332, 3624660, 3626903

Submitter: Ventspils Free Port Authority, Jāņa street 19, Ventspils

Expected activity: “Reconstruction of access roads to the Ventspils port terminals”

- Construction of new ferry terminal; construction of new access road from Pils street till the new ferry terminal (Prāmju street); construction of new berth No.17; reconstruction of Sarkanmuižas dam; reconstruction of road surface (coverage) in the section from administration border of city till Riga motorway (Kurzemes street); reconstruction of traffic bridge over the Venta river; new traffic bridge beside the existing railroad bridge and access roads of it.

Place of activities: territory of VFP

Substantiation: application of Ventspils Free port Authority, resolution of VPVB No.351

  - Requirements of environmental protection:

  - Reconstruction of bridge must be expected according technical regulations issued by Ventspils Regional Environmental authority No. 4-21.1/114 as of 18th August, 2003.

  - If there is expected demolition of existing buildings (including roofs with coverage made from asbestos – cement), management of debris containing asbestos must be according regulation No. 425 of Cabinet of Ministers (MK) as of October 2, 2001 “Regulations on environmental pollution caused by asbestos and asbestos products and management of asbestos waste”. Hazardous waste must be delivered to the management companies which have received permission of management of corresponding kind of waste.

  - Must be expected collection and utilisation of waste occurred during construction and demolition works according law of LR as of December 14, 2000 “Law on waste management” and according regulation Nr. 342 of Cabinet of Ministers (MK) as of May 17, 2005 “Submission, prolongation and order of annulment of waste management permissions.”

  - Must be expected water supply according requirements of regulation Nr. 38 of MK as of February 1, 2000 “LBN 222-99 external networks and constructions of water supply”.

  - Must be expected establishment of meter of used water according regulations Nr. 736 of MK as of December 23, 2003 “Regulations on permission of usage of water resources”.

  - To expect drainage of rain and household waste water into the existing sewage network according regulations Nr. 214 of MK as of June 15, 1999 “LBN 223-99 External network and constructions of sewerage”

  - Must be expected treatment of waste water before draining to the sewage network according regulation nr. 34 of MK as of January 22, 2002 “Regulations on emission into water of contaminating substances”.

  - Must be expected activities that during construction process there will not be exceeded allowable values of noise emission according regulations Nr. 163 of MK as of April 23, 2002 “Regulations on noise emission from equipment used outside premises” as well as the stated allowable noise in the territory of living and public buildings according regulations Nr. 598 of Mk as of July 13, 2004 “Regulations on norms of acoustic noise in the premises and territory of living and public buildings”.

  - During construction works connected with the activities in the Venta river the construction works must be expected outside the spawning time (according regulations Nr. 3 of MK as of January 2, 2001 “Regulations on industrial fishing in the inland waters” part V, paragraph 27 and 28.

  - According amendments of Law on Fishery of LR as of February 17, 2000; paragraph 26, point 3, before inception of works there must be carried out expertise on fishery for reason to determine impact on fish resources and amount of ecological system, work substantiation, possible losses and amount and kind of compensation.

  - Caused losses to the fish resources must be calculated according regulations Nr. 188 of MK “Determination and order of compensation of caused damage to the fish resources due to economical activity”.

  - Construction of new berth No.17 has to be made according regulation Nr. 363 of MK as of 28 October, 1997 “Special construction regulations of hydrotechnical constructions of ports”.

  - The employer must provide informing of owners and mangers of properties beside the territory of activity according stated in regulations Nr. 91 of MK as of February 17, 2004 “Order, how regional environmental authorities issues technical regulations for the expected activity which does not need EIA”.

  - Expecting to have works in the basin of port there must be received conditions from Sea Environmental authority.

  - Till start of activities to receive permission according regulations Nr. 294 of MK as of July 9, 2002 “Order, how to declare contaminating activities of A, B and C categories and issued permissions for implementation of contaminating activities of A, B and C categories”.

  - Evaluated documentation:

2.1 Application of expected activity on 97 pages;

2.2 Initial evaluation on 4 pages;

2.3 Additional info prepared by the submitter on 1 page;

2.4 Experts opinion on ___pages;

2.5 Information granted by state and institutions of municipalities on 35 pages;

2.6 Written proposals submitted by society on ___ pages.

The determined requirements of environmental protection in the technical regulations can be amended according regulations Nr.91 of MK as of February 17, 2004 “Order how regional environmental authorities issues technical regulations for the expected activity which does not need EIA”.

These technical regulations can be arbitrated in one month time in the office of State Environmental Supervision office, at Rūpniecības street 23, riga, LV – 1045.

Application on arbitration of Administrative Act according Law on Administrative process (as of November 14, 2001 with amendments on January 23, 2004); paragraph 77 must be submitted to the Ventspils Regional Environmental authority, Dārza street 2, Ventspils, LV – 3601.

Deputy director 							V.Borovikova

July 29, 2005.

Ventspils City Council Order copy

Translation from Latvian

Republic of Latvia

VENTSPILS CITY COUNCIL

ORDER

Ventspils

April 8, 2005									     Nr. 821

On approval of Planning and Architectural

Task Nr.34

On the bases of application Nr. 1-4.1/268 as of February 23, 2005 submitted by Ventspils Free port Authority and all the annexed materials, and according Binding construction regulation of Ventspils City, points 1.1.2.,

determine:

- To submit construction permission to VFPA for the object “Construction of new traffic bridge over the Venta river, Ventspils”.

- To approve Planning and Architectural task Nr. 34 made by Architectural ans city construction department of the city council for the object of VFPA “Construction of new traffic bridge over the Venta river, Ventspils”.

Chairman of the Council 						A.Lembergs

J.Vītoliņš

M.Bože

3601164

Ventspils City Council planning and arcitectural task nr.34 copy

Translation from Latvian

Republic of Latvia

VENTSPILS CITY COUNCIL

ARCHITECTURAL AND CITY CONSTRUCTION DEPARTMENT

Jūras street 36, Ventspils, LV – 3601; phone 3601162; fax: 3601118; e-mail: apn@ventspils.gov.lv

Ventspils

Prepared on March 30, 2005

Issued to Ventspils Free Port Authority

PLANNING AND ARCITECTURAL TASK NR 34

Construction of new traffic bridge over the Venta river, Ventspils

- Characteristics of plot

| 1.1 | Owner of plot | agrees to municipality of Ventspils city |
| --- | --- | --- |
| 1.2 | Rights of usage | under possession of Ventspils Free Port Authority |
| 1.3 | Location and situation of the territory, buildings and constructions on it | - location is industrial construction zone of the city, at the territory of Ventspils Free Port; - location of the new bridge is beside the railroad bridge, at the side of the existing traffic bridge, in the free territory |
| 1.4 | Special circumstances | - location beside the existing railroad bridge; - important object of traffic infrastructure |
| 1.5 | Kind of construction | Newly erected building |
| 1.6 | Stages of designing | - stage of sketch project (SP); - stage of technical project (TP); - order of coordination see chapter nr. 3and nr.4 |

- Conditions of zoning

| 2.1 | Kind of usage of plot, border of work | - traffic bridge which connects both banks of the river Venta; - location see in appendix 1 |
| --- | --- | --- |
| 2.2 | Limitations, special requirements, objects to be designed: |  |
|  | - to respect the existing and designed locations of surrounding streets and traffic flows (detailed information can be received at p/i “Komunālā pārvalde”, phone: 3624269); - the project must be worked out according Terms of Reference, laws and regulations of LR and Ventspils city being in force; - to respect red lines of access streets of the bridge; - during designing of bridge to respect that beside there is located the existing as well as perspective, possibly, railroad infrastructure to be constructed; - to respect height remark of railroad surface; - in the design of bridge to expect: - construction of bridge with 2 traffic lines and pedestrian sidewalk at the side of existing traffic bridge; location of pilons of new bridge – on one axle with the existing pilons of railroad bridge by specifying optimal fixation and respecting red lines of access roads of bridge. This condition can not be observed only in case if it can be technically-economically sustained; - lighting of bridge; - drainage of rain water from the carriageway of the bridge; - organisation of traffic flow. |  |
| 2.3 | Indices of capacity: | - according Latvian Construction Norms and programme given by the Client |
| 2.4 | Common regulations for the improvement of the territory: |  |
| 2.4.1 | Greening, improvement | - to indicate in detailed way the used materials, elements of improvement (small forms of architecture in the framework of project conception – lighting facilities, parapets, dust-bins, etc.); |
| 2.4.2 | Carriageway, sidewalks | - according the Terms of Reference of the Client; - widths and radiuss of bends according the requirements of LAD and LBN; - to assure accessibility for the persons of special needs |
| 2.4.3 | Lighting | To expect establishment of new lighting devices (with yellow light0 according the Terms of Reference and requirements of LBN; |
| 2.4.4 | Vertical planning | To design in the whole territory |
| 2.4.5 | Coverage of carriageway and sidewalks | - for the carriageway of bridge – asphalt-concrete; - for the sidewalk – to expect coloured concrete paving stone of thickness of 6cm; - in the stage of TP to work out detailed composed matters of colour and ornaments of concrete paving stones |

- Conditions of work out and coordination of construction project

| 3.1 | Water supply, sewerage | To demand technical regulations from PSIA “Udeka’’ |
| --- | --- | --- |
| 3.2 | Drainage of rain and surface water | Closed, according requirements of Ventspils regional Environmental Authority and TR of PSIA “Udeka” |
| 3.3 | Carriageway | According laws and regulations of LR, requirements of Latvian road management, p/i “Komunālā pārvalde” |
| 3.4 | Electrical supply | According requirements of TR of VAS Latvenergo, RET Ventspils ETR |
| 3.5 | Heating supply | No |
| 3.6 | Gas supply | No |
| 3.7 | Telecommunications | No |
| 3.8 | Cable television | No |

Remarks:

- for designing use new topographical survey (not older than 1 year) according the requirements of LBN 005-99, in which are indicated underground communications, borders of plots and red lines of the streets. The topographical plan must be coordinated with SJSC “Latvenego” Western Electrical Network, SIA “Lattelecom”, PSIA “Udeka”, State Land service, Architectural and city construction department (APN) network engineer, engineer of council GIS and manager of APN territorial planning and land survey department;

- new cable lines must be located within the borders of red lines;

- the construction project must be formatted according the requirements of LBN 202-01 “Contents and formatting of construction project”;

- the construction project has to be worked out in digital form, LKS – 92TM coordinate system, in Baltic height system;

- parts of Technical design and general plan must be submitted in electronic format (“dwg or dxf” files, in coordination system of LKS – 92TM, the designed communications are located in separate layers) and in paper format (copy for VCC archive must be bound in hard covers), (detailed information can be received at APN, phone: 3601069, network engineer N.Māls).

| 3.9 | Special conditions determined by the following services |  |
| --- | --- | --- |
| 3.9.1 | State Fire fighting and rescue service | According the issued instructions and TR |
| 3.9.2 | Ventspils Regional Environmental authority | According the issued instructions and TR |
| 3.9.3 | Sea Environmental authority | According the issued instructions and TR |
| 3.9.4 | Latvian road management | According the issued instructions and TR |
| 3.9.5 | SJSC “Latvian railroads” | According the issued instructions and TR |

- Conditions of coordination – acceptance of construction project

| 4.1 | Coordination of Sketch project | At Architectural and city construction department, Jūras street 36, Ventspils; beforehand coordinating with VFPA, p/i “Komunālā pārvalde”, Environmental supervision department of VCC and with institutions and services mentioned in point 3 (copy of SP for VCC must be bound in hard covers) |
| --- | --- | --- |
| 4.2 | Accept of technical project | At Architectural and city construction department, Jūras street 36, Ventspils after coordination with institutions and services mentioned in point 4.1 and after reception of positive conclusion of construction expertise |
| 4.3 | Construction permission | At Construction administrative inspection, Jūras street 36, Ventspils |
| Deputy manager of APN |  | Mr. Mareks Noviks |
| City architect in questions of development of infrastructure |  | Mrs. Ieva Vanaga |
| Network engineer |  | Mr. Normunds Māls |
| Landscpae architect |  | Mrs. Anita Neilande |
| Conditions of coordination of this construction project are valid two years and can be prolonged or renewed after the mentioned term |  |  |

Translation from Latvian

Republic of Latvia

VENTSPILS CITY COUNCIL

ORDER

Ventspils

December 7, 2005							Nr. 3344

On changes in the Planning and Architectural

Task Nr.34 “Construction of new traffic bridge

over the Venta river, Ventspils”

Evaluating solutions of sketch project and proposal of project author of object “Construction of new traffic bridge over the Venta river, Ventspils”,

determine:

During the work out of the construction project, to observe the designing requirements defined in the Planning and Architectural task Nr.34 “Construction of new traffic bridge over the Venta river, Ventspils” except point 2.2.6, which has to be expressed in the following wording:

| 2.2 | Limitations, special requirements, objects to be designed |
| --- | --- |
|  | - in the design of bridge to expect: - construction of bridge with 2 traffic lines and pedestrian sidewalk with width of 1.5m at the both sides of the bridge; location of pilons of new bridge – on one axle with the existing pilons of railroad bridge by specifying optimal fixation and respecting red lines of access roads of bridge. This condition can not be observed only in case if it can be technically-economically sustained; - lighting of bridge; - drainage of rain water from the carriageway of the bridge; - organisation of traffic flow |

Chairman of the council 							A.Lembergs

J.Vītoliņš

M.Bože

I.Vanaga

3601168

Translation from Latvian

Republic of Latvia

VENTSPILS CITY COUNCIL

ORDER

Ventspils

January 17, 2007							Nr. 121

On confirmation of Planning and

Architectural task Nr.304 as of December 27, 2006

Considering the application Nr. 3-4.1/1847 submitted by Ventspils Free Port Authority on December 8, 2006 and the annexed materials, on the bases of Ventspils city binding regulation on usage of territory and on construction, point 2.8.1,

determine:

To confirm Planning and Architectural task nr.304 prepared by Architectural and city construction department on December 27, 2006 for the object of VFPA “Reconstruction of Rūpniecības street in the section from the drive to fuel station “Statoil” till the railroad level crossing, Ventspils”.

1 Deputy of Executive director 							J.Vītoliņš

M.Bože

3601164

Ventspils City Council planning and arcitectural task nr.304 copy

Translation from Latvian

Republic of Latvia

VENTSPILS CITY COUNCIL

ARCHITECTURAL AND CITY CONSTRUCTION DEPARTMENT

Jūras street 36, Ventspils, LV – 3601; phone 3601162; fax: 3601160; e-mail: apn@ventspils.gov.lv

Ventspils

Prepared on December 27, 2006

Nr.9-10/2824

Issued to Ventspils Free Port Authority

PLANNING AND ARCITECTURAL TASK NR 304

Reconstruction of Rūpniecības street in the section from the drive to fuel station “Statoil” till the railroad level crossing, Ventspils

- Characteristics of plot

| 1.1 | Cadastre number of the plot | ----------------------------------- |
| --- | --- | --- |
| 1.2 | Owner or user of plot | - part of the designed territory agrees to the Municipality of Ventspils city; - part (plot at Rūpniecības street 29, Ventspils part ~ 850 m2 ) – property of municipality of Ventspils city |
| 1.3 | Rights of usage | - Agrees to municipality of Ventspils city; - certification of Land Register |
| 1.4 | Area of the plot | - length of the streets to be designed ~ 1250m |
| 1.5 | Location and situation of the territory, buildings and constructions on it | - location is industrial construction zone of the city, at the territory of Ventspils Free Port |
| 1.6 | Special circumstances | -------------------------------- |
| 1.7 | Kind of land usage | Territory of objects of traffic infrastructure – land under the streets of common use. |
| 1.8 | Limitations | -------------------------------------- |

- Conditions of construction project

| 2.1 | Kind of construction | - reconstruction |
| --- | --- | --- |
| 2.2 | Stages of construction design | - stage of Technical project (TP); - the project must be worked out according the coordinated Sketch project on April 7, 2006 (Nr.24) of object “Access roads of left bank for the new bridge over the Venta river, Ventspils” and coordinated solutions of technical project on January 27, 2005 (Nr.22) of object “Reconstruction of Rūpniecības street in the section from Durbes street till Liepāja railroad, Ventspils”, author of the project SIA “Office of E.Daniševskis”, information can be received at p/i “Komunālā pārvalde”, phone 3624269); - border of works see in the scheme of Annex 1 |
| 2.3 | Basic conditions of construction | --------------------------------------- |
| 2.4 | Basic conditions of composition |  |
| 2.4.4. | Drive in and entrances | - Location of drive in according the projects mentioned in point 2.2 and according directions of the client; - to expect drive in to the plot at Rūpniecības street 25 right in front of the drive in to the plot at Rūpniecības street 14, Ventspils. Drive to the plot at Rūpniecības street 14, Ventspils must be expected according project solutions of the work out stage of object “Production plant of large scale fibreglass products on the lease plot at Rūpniecības street 14, Ventspils” (tenant of plot SIA “Valmiera – Andren”, contact phone 42 21047) (see appendix 2); - to expect drive to the plot at Rūpniecības street 27, respecting the existing project solutions of the work out stage of the object “Construction of surrounding infrastructure of the production building at Rūpniecības street27, Ventspils” (information can be received at VFPA) (see appendix 3) |
| 2.5 | Basic conditions of designing of constructions |  |
| 2.5.2 | Load bearing constructions | - according the requirements of laws and regulations being in force |
| 2.6. | Conditions of external finish | ----------------------------------------- |
| 2.7 | Common regulations for the improvement of the territory: |  |
| 2.7.1 | Greening, improvement | - to work out construction project of street section in the borders of red lines, expecting carriageway and side walk at the one side of the carriageway, as well as stops of public transport; - to design side walks at the both sides of the carriageway for the section from Rūpniecības street towards the direction of new traffic bridge over the Venta river (till the territory of drive in of the lease plot at Rūpniecības street 27); at the 1st stage to expect construction of side walk at the one side of the carriageway (NW); - to expect new greening of the streets. During the project work out on kind of greening please consult with scenery architect Mrs. A.Neilande (phone 3601163); |
| 2.7.2 | Fencing | --------------------------------- |
| 2.7.3 | Lighting | - in the project there have to be enclosed lighting project of the street section; - the light fittings must be located at the same distance from the axes of the street; |
| 2.7.4 | Vertical planning | - to work out for the whole territory; |
| 2.7.5 | Coverage of carriageways and side walks | - for the carriageway, drives to the plots, stops of the public transport to expect coloured concrete paving stone of thickness of 8 cm; - for the sidewalks and platforms of stops for the public transport to expect coloured concrete paving stone coverage of thickness of 6 cm; - to work out detailed composed matters of colour and ornaments and joints of different covers for carriageways, drives, parking places, sidewalks by using the accepted (Nr.22) on January 27, 2005 technical project solutions of object “Reconstruction of Rūpniecības street in the section from Durbes street till Liepāja railroad, Ventspils”, author of the project SIA “Office of E.Daniševskis” |
| 2.7.6 | Utilization of debris, recycling or permission to use dump | - To indicate the unnecessary amount of ground (if there occurs such ones) which have to be transported to the “Adventure park”, beforehand concluding contract with p/i “Komunālā pārvalde” (contact phone number 36 24269) |
| 2.8 | Requirements of environmental accessibility |  |
| 2.8.1 | Territory | - to provide environmental accessibility for the persons of special needs |

- Technical regulations

| 3.1 | Water supply, sewerage, drainage of rain water | -according the coordinated Sketch project on April 7, 2006 (Nr.24) of object “Access roads of left bank for the new bridge over the Venta river, Ventspils” and coordinated solutions of technical project on January 27, 2005 (Nr.22) of object “Reconstruction of Rūpniecības street in the section from Durbes street till Liepāja railroad, Ventspils”, author of the project SIA “Office of E.Daniševskis”, information can be received at p/i “Komunālā pārvalde”, phone 3624269); |
| --- | --- | --- |
| 3.2 | Streets and roads | - according the coordinated Sketch project on April 7, 2006 (Nr.24) of object “Access roads of left bank for the new bridge over the Venta river, Ventspils” and coordinated solutions of technical project on January 27, 2005 (Nr.22) of object “Reconstruction of Rūpniecības street in the section from Durbes street till Liepāja railroad, Ventspils” |
| 3.3 | Electric supply | - according the coordinated Sketch project on April 7, 2006 (Nr.24) of object “Access roads of left bank for the new bridge over the Venta river, Ventspils” and coordinated solutions of technical project on January 27, 2005 (Nr.22) of object “Reconstruction of Rūpniecības street in the section from Durbes street till Liepāja railroad, Ventspils”, author of the project SIA “Office of E.Daniševskis”, information can be received at p/i “Komunālā pārvalde”, phone 3624269); |
| 3.4 | Gas supply | --------------------------------------- |
| 3.5 | Heating | --------------------------------------- |
| 3.6 | Telecommunications | - according the coordinated Sketch project on April 7, 2006 (Nr.24) of object “Access roads of left bank for the new bridge over the Venta river, Ventspils” and coordinated solutions of technical project on January 27, 2005 (Nr.22) of object “Reconstruction of Rūpniecības street in the section from Durbes street till Liepāja railroad, Ventspils”, author of the project SIA “Office of E.Daniševskis”, information can be received at p/i “Komunālā pārvalde”, phone 3624269); |
| 3.7 | Other communications | ------------------------------ |

Remarks:

- for designing use new topographical survey (not older than 1 year) according the requirements of LBN 005-99, with scale M 1:500 in which are indicated borders of plots and red lines of the streets. The topographical plan must be coordinated with SJSC “Latvenego” Western Electrical Network, SIA “Lattelecom”, PSIA “Udeka”, State Land service, Architectural and city construction department (APN) network engineer, engineer of council GIS and manager of APN territorial planning and land survey department.

- The general layout must be worked out in scale M 1:250;

- the construction project must be formatted according the requirements of LBN 202-01 “Contents and formatting of construction project”;

- the construction project has to be worked out in digital form, LKS – 92TM coordinate system, in Baltic height system.

4. Special regulations

| 4.1 | Environmental requirements | ----------------------------------------- |
| --- | --- | --- |
| 4.2 | Hygienic requirements | ----------------------------------------- |
| 4.3 | Fire safety requirements | according the coordinated Sketch project on April 7, 2006 (Nr.24) of object “Access roads of left bank for the new bridge over the Venta river, Ventspils” and coordinated solutions of technical project on January 27, 2005 (Nr.22) of object “Reconstruction of Rūpniecības street in the section from Durbes street till Liepāja railroad, Ventspils”, author of the project SIA “Office of E.Daniševskis”, information can be received at p/i “Komunālā pārvalde”, phone 3624269); |
| 4.4 | Safety requirements of constructions | according the requirements of laws and regulations being in force |
| 4.5 | Requirements of protection of cultural monuments | ------------------------------------------ |
| 4.6 | Other requirements | - to respect technical project solutions accepted on November 9, 2004 (nr.349) of object “Reconstruction of durbes street in the section form Dīķu street till Rūpniecības street”; - till reception of construction permission to work out work organisation project, as well as to inform the owners, tenants and managers of the surrounding territory for the planned construction works of the section of Rūpniecības street. |
| 4.7 | Permission of tree cutting | In case of necessity cutting of separate trees must be coordinated with Commission of greenery maintaining of VCC (Leader of commission Mrs. A.Neilande, phone 3601163) |
| 4.8 | Other permissions | ----------------------------------- |

- Additional materials for public construction discussions -----------------

6. Conditions of coordination-acceptance of construction project

| 6.1 | Coordination of sketch project | ---------------------- |
| --- | --- | --- |
| 6.2 | Accept of technical project | - the technical project must be submitted for acceptance to the following institutions: APN, Jūras street 36, Ventspils – beforehand coordinating it with the client, p/i “Komunālā pārvalde”, Environmental supervision department of City Council, PSIA “Udeka”, VAS “Latvenergo”, RET Ventspils ETR, VAS “Latvenergo “ “Augstsprieguma tīkls”, SIA “Lattelecom”, VUGD, VAS “Latvijas dzelzceļš”, VAS “Latvijas valsts ceļi”, SIA “Valmiera – Andren”, and SIA “E.Daniševska birojs”. |
| 6.3 | Reception of construction permission | - At Construction administrative inspection, Jūras street 36, Ventspils; - according point 4.6 |

Planning and Architectural task is valid for 2 years since acceptance of it.

Director of APN							Mr. Māris Bože

City architect in questions of development

of infrastructure							Mrs. Ieva Vanaga

Network engineer of APN department					Mr. Normunds Māls

This administrative act in two month time after coming into force can be arbitrated in order stated in Law on Administrative process, according paragraph 76, part 2 and paragraph 79 part 1, by submitting the corresponding application to the VCC addressing it to the Chairman of the VCC.